Communication Project

Ziad Sherif Muhammed		BN:27 	Sec:1

Eslam Ashraf	 Ibrahim		BN:13		Sec:1

- Our Work Explanation:

Firstly, we read three signals with different sounds and same sampling frequencies then, we choose the first channel for our messages and find the length of each one along with max length in-order to be able to sum all modulated signal of each input. We make all audios have the same length by adjusting all of them by adding zero to make have same length. Secondly got time and frequencies intervals. Then we calculated omega (ω1, ω2) by ω=2*pi*const in frequency domain. Then we got carrier in cos and sin domain

Carrier Signal One=cos (2*pi* ω1)

Carrier Signal Two=sin (2*pi* ω2)

Modulation:

- Calculate modulated signal by multiplying signal and carrier

- Calculate Fourier transform of modulated signal returned from step 1

- Calculate phase of modulated signal returned from step 1

- Sum all modulated signals of all audios

- FFT to summation of modulated signals

- Calculate phase of modulated signals

- Calculate frequency band pass which is used in de-modulation

De-modulation:

- Calculate demodulated signal by multiplying carrier and modulation signal

- Calculate low pass filter used frequency sampling.

- Perform demodulation three times with phase shifts of 10, 30, 90 degrees for both carriers note (before each phase shift calculate carrier phase).

- perform demodulation two times with a local carrier frequency that is different by 2 Hz and 10 Hz from its carrier frequency.

- Required results and answers to questions:

- Required figures:

- Codes:

%% Read Voice

[signal_1, frequency_sampling1] = audioread('signals/esoo.wav');

[signal_2, frequency_sampling2] = audioread('signals/ziad.wav');

[signal_3, frequency_sampling3] = audioread('signals/mohey.wav');

%% Sum first and second channel

signal_1 = signal_1(:, 1)+signal_1(:, 2);

signal_2 = signal_2(:, 1)+ signal_2(:, 2);

signal_3 = signal_3(:, 1)+signal_3(:, 2);

%% frequency Sampling

frequency_sampling = frequency_sampling1;

%% set maxuim length between 3 signals

length_1 = length(signal_1);

length_2 = length(signal_2);

length_3 = length(signal_3);

N = max([length_1, length_2, length_3]);

%% make 3 signals be same

signal_1 = [signal_1;zeros(N-length_1, 1)];

signal_2 = [signal_2;zeros(N-length_2, 1)];

signal_3 = [signal_3;zeros(N-length_3, 1)];

%% get time and frequencyuency interval

t = linspace(0, N/frequency_sampling, N);

frequency_d = frequency_sampling/2;

frequency = -frequency_d : frequency_sampling/N: frequency_d - frequency_sampling/N;

%% frequencyuency Domin

signal_1_fft = fftshift(fft(signal_1));

signal_2_fft = fftshift(fft(signal_2));

signal_3_fft = fftshift(fft(signal_3));

phase_signal_1 = unwrap(angle(signal_1_fft));

phase_signal_2 = unwrap(angle(signal_2_fft));

phase_signal_3 = unwrap(angle(signal_3_fft));

%% Carrie in frequencyuency Domin

Carrier_frequencyuency_1 = 5000;

Carrier_frequencyuency_2 = 3*5000;

WC_1 = 2*pi * Carrier_frequencyuency_1;

WC_2 = 2*pi * Carrier_frequencyuency_2;

%% Carrier in time Domain

carrier_signal_1 = cos(WC_1 * t);

carrier_signal_2 = cos(WC_2 * t);

carrier_signal_3 = sin(WC_2 * t);

phase_carrier_signal_1 = unwrap(angle(fftshift(fft(carrier_signal_1))));

phase_carrier_signal_2 = unwrap(angle(fftshift(fft(carrier_signal_2))));

phase_carrier_signal_3 = unwrap(angle(fftshift(fft(carrier_signal_3))));

%% Modulation

modulatedSignal_t1 = signal_1' .* carrier_signal_1;

modulatedSignal_t2 = signal_2' .* carrier_signal_2;

modulatedSignal_t3 = signal_3' .* carrier_signal_3;

modulated_signal_f1 = fftshift(fft(modulatedSignal_t1));

modulated_signal_f2 = fftshift(fft(modulatedSignal_t2));

modulated_signal_f3 = fftshift(fft(modulatedSignal_t3));

phase_modulation_1 = unwrap(angle(modulated_signal_f1));

phase_modulation_2 = unwrap(angle(modulated_signal_f2));

phase_modulation_3 = unwrap(angle(modulated_signal_f3));

%% Add modulated signals

Modulated_Signal = modulatedSignal_t1 + modulatedSignal_t2 + modulatedSignal_t3;

Modulated_Signal_f = fftshift(fft(Modulated_Signal));

phase_mod = unwrap(angle(Modulated_Signal_f));

frequncy_BandPass = Carrier_frequencyuency_1;

%% Synchronous Modulation for 3 signals

FileNames=["Out_1" "Out_2" "Out_3"];

Carries=[carrier_signal_1 ;carrier_signal_2; carrier_signal_3];

for i = 1:3

LowPass=demodulation(Modulated_Signal,frequncy_BandPass,FileNames(i), Carries(i,:,:), frequency_sampling);

fftSignal = fftshift(fft(LowPass));

N=(length(fftSignal)/2);

frequency = (-1*N:N-1);

phase=unwrap(angle(fftSignal));

plot_signal(t, frequency.',phase,LowPass, abs(fftSignal),i+6, strcat('Deodulated Signal ',num2str(i,' %2d')));

end

%%

% Phase Shift: 10

Carries_phase_10 = CarriersPhase(Carrier_frequencyuency_1, Carrier_frequencyuency_2, 10,t);

FileNames_phases_10=["Out_1_phase_10" "Out_2_phase_10" "Out_3_phase_10"];

for i = 1:3

demodulation(Modulated_Signal,frequncy_BandPass,FileNames_phases_10(i), Carries_phase_10(i,:,:), frequency_sampling);

end

% Phase Shift: 30

Carries_phase_30 = CarriersPhase(Carrier_frequencyuency_1, Carrier_frequencyuency_2, 30,t);

FileNames_phases_30=["Out_1_phase_30" "Out_2_phase_30" "Out_3_phase_30"];

for i = 1:3

demodulation(Modulated_Signal,frequncy_BandPass,FileNames_phases_30(i), Carries_phase_30(i,:,:), frequency_sampling);

end

% Phase Shift: 90

Carries_phase_90 = CarriersPhase(Carrier_frequencyuency_1, Carrier_frequencyuency_2, 90,t);

FileNames_phases_90=["Out_1_phase_90" "Out_2_phase_90" "Out_3_phase_90"];

for i = 1:3

demodulation(Modulated_Signal,frequncy_BandPass,FileNames_phases_90(i), Carries_phase_90(i,:,:), frequency_sampling);

end

%%

% local carrier frequencyuency different than Fc by 2 Hz in Demodulation

Carries_shift_2 = CarriersDifferentFc(Carrier_frequencyuency_1, Carrier_frequencyuency_2, 2,t);

FileNames_shift_2=["Out_1_shift_2" "Out_2_shift_2" "Out_3_shift_2"];

for i = 1:3

demodulation(Modulated_Signal,frequncy_BandPass,FileNames_shift_2(i), Carries_shift_2(i,:,:), frequency_sampling);

end

%  local carrier frequencyuency different than Fc by 10 Hz in Demodulation

Carries_shift_10 = CarriersDifferentFc(Carrier_frequencyuency_1, Carrier_frequencyuency_2, 10,t);

FileNames_shift_10=["Out_2_shift_10" "Out_2_shift_10" "Out_3_shift_10"];

for i = 1:3

demodulation(Modulated_Signal,frequncy_BandPass,FileNames_shift_10(i), Carries_shift_10(i,:,:), frequency_sampling);

end

% Ploting  Signals

plot_signal(t, frequency.', phase_signal_1,signal_1, abs(signal_1_fft),1, 'Signal 1');

plot_signal(t, frequency.', phase_signal_2,signal_2, abs(signal_2_fft),2, 'Signal 2');

plot_signal(t, frequency.', phase_signal_3,signal_3, abs(signal_3_fft),3, 'Signal 3');

% Ploting Modulated Signals

plot_signal(t, frequency.',phase_modulation_1,modulatedSignal_t1, abs(modulated_signal_f1),4, 'Modulated Signal 1');

plot_signal(t, frequency.', phase_modulation_2,modulatedSignal_t2, abs(modulated_signal_f2),5, 'Modulated Signal 2');

plot_signal(t, frequency.', phase_modulation_3,modulatedSignal_t3, abs(modulated_signal_f3),6, 'Modulated Signal 3');

function plot_signal(length_t, f,angle, time, freq,counter, Time)

figure(counter)

subplot(3, 1, 1)

plot(length_t, time)

xlabel('time')

%title of time Domain

title(strcat(Time, ' signal in time Domin'))

subplot(3, 1, 2)

plot(f, freq)

xlabel('f')

%title of frequency Domain

title(strcat(Time, ' Signal in frequency Domain'))

subplot(3, 1, 3)

plot(f, angle)

ylabel('phase')

xlabel('f')

%title of phase

title(strcat(Time, ' Phase'))

end

function lpf=demodulation(signal_modulation, frequncy_BandPass, out_name, carrier_signal, frequency_signal_sampling)

demodulationdSignal = 2 * (signal_modulation .* carrier_signal);

lpf = lowpass(demodulationdSignal, frequncy_BandPass, frequency_signal_sampling);

audiowrite(strcat("Output_signals/", out_name, '.wav'), lpf, frequency_signal_sampling);

end

function Array_of_carries = CarriersPhase(Carrier_frequencyuency_1, Carrier_frequencyuency_2, deg,time)

carry_1 = cos((2*pi * Carrier_frequencyuency_1 * time) + ((deg * pi) / 180));

carry_2 = cos((2*pi * Carrier_frequencyuency_2 * time) + ((deg * pi) / 180));

carry_3 = sin((2*pi * Carrier_frequencyuency_2 * time) + ((deg * pi) / 180));

Array_of_carries=[carry_1; carry_2; carry_3];

end

function Array_of_carries  = CarriersDifferentFc(Carrier_frequencyuency_1, Carrier_frequencyuency_2, frequency_dc,t)

carry_1 = cos(2*pi * (Carrier_frequencyuency_1 + frequency_dc) * t);

carry_2 = cos(2*pi * (Carrier_frequencyuency_2 + frequency_dc) * t);

carry_3 = sin(2*pi * (Carrier_frequencyuency_2 + frequency_dc) * t);

Array_of_carries=[carry_1; carry_2; carry_3];

end